Technical Specification DVB cable Receiver

TechnoTrend

Performance on your side.

TT-micro@ C254

based on TT-micro® family

Technical Specifications

Basics

DVB cable receiver compliant to EN 300 429

Embedded Nagravision CAKUNM

Video and Audio out via SCART

Digital Audio out via optical S/PDIF

Analogue Audio out (left/right) via RCA

Reed-Solomon decoding support (EN 300 429)

MPEG2 Transport Stream support: ISO/IEC 13818-1

DVB-Video/-Audio support: ISO/IEC 13818-2, -3

Update over DVB cable network

DVB-Subtitling

Teletext Browser

RF specification

Frequency range: 47...862 MHz (VHF/UHF)

QAM 16/32/64/128/256 demodulation support (EN 300 429)

Symbol rate: 1.7...7 Msymbols/s

VHF/UHF support, 8 MHz bandwidth

Input connector: IEC 60169-2 female

Input termination: 75 Ω

Loop-Through connector: IEC 60169-2 male

Loop-Through output termination: 75 Ω

Input / Output specification

•     RF In/Out:

o    Input connector: IEC 60169-2 female (75 Ω)

o    Loop-Through connector: IEC 60169-2 male (75 Ω)

•	2x SCART out (VCR & TV) accordingly to EN 50049-1:

o    TV SCART out: CVBS, Y/C, RGB, Audio

o    VCR SCART out: CVBS, Y/C, Audio

o    VCR SCART in (pass to TV SCART): CVBS, Y/C, RGB, Audio

o    Switching signal: at PIN8 0/6/12 V; at PIN16 0/1-3 V

•	RCA (Cinch) out:

o    Digital Audio (PCM stereo, AC3) out via optical S/PDIF o    Analogue Audio out (left/right) via RCA

TechnoTrend AG © 2006

Page 1 of 2

1.2 TT

Technical Specification DVB cable Receiver

TechnoTrend

.		Performance on your side.

•	Ethernet RJ45 connector

STB mode functionality

- Pay- and Free-TV support

- DVB cable TV and Radio reception

- Multi language support

- Easy-to-use graphical user interface based on widgets

- Context referred help function

- Automatic and manual channel scan

- Separated program list for TV and Radio

- Up to 4 different favourite lists

- Up to 2000 programs storable

- Electronic Program Guide: present/following EPG plus additional 7-Day-EPG

- Aspect ratio: 4:3,16:9,2.21:1 (PAL only)

- Wide Screen Signalling (WSS)

- Teletext re-insertion (VBI)

- Easy-to-use Timer functionality

Power Supply

- Power supply 230VAC/50Hz to 12 VDC

- Operation power consumption less 10 W

Accessories

- Receiver (ca. 250x135x35 [mm])

- Power Supply Unit

- SCART cable

- Coaxial Cable

- Remote Control inclusive batteries

- Cardboard box

- Instruction manual: A5 format, black/white printed in German, English and Turkish language

Standard and Regulation Compliance

- DVB cable standard EN 300 429

- MPEG2 Transport Stream support to ISO/IEC 13818-1

- DVB-Video/-Audio support to ISO/IEC 13818-2, -3

- Teletext in VBI according to ETS 300 472

- DVB-Subtitling according to ETS 300 743

- Safety and EMC regulations: EN 55013, EN 55020 and EN 60065

TechnoTrend AG © 2006	Page 2 of 2	1.2 TT